Concept Plan
Unitree Go2 ROS 2 / Gazebo -> Transport -> Gateway -> HLA

Working baseline: Ubuntu 22.04, ROS 2 Humble, Gazebo 11.10.2
Strategy: start from a clean architecture, reuse only the useful ideas from the earlier prototype
Project objective: make a simulation of the Unitree Go2 robot visible inside an HLA distributed simulation

Target pipeline Gazebo (Go2) <-> ROS 2 <-> adapter (VehicleState/RobotState + sensors) <-> transport <-> gateway <-> HLA (FOM)

Prepared as a working reference for project iteration.

# 1. Project goal

- Build a working end-to-end path from Unitree Go2 simulation in Gazebo and ROS 2 into an HLA federation through a clean adapter, transport layer, and gateway.

- Use the diff_robot / MQTT prototype only as a source of ideas; do not inherit its architecture blindly.

- Establish one clean submission baseline first, then extend it with sensors, reverse commands, measurements, and documentation.

# 2. Guiding principles

- Work in layers. First prove the Go2 simulation works locally, then add the adapter, then transport, then gateway, and only then HLA publication.

- Prefer a small working demo at each stage over parallel partially working branches.

- Keep the internal state model independent from any single transport protocol.

- Treat MQTT as the first transport baseline and gRPC as a later extension or comparison path.

- Use /odom as the primary first state source because it matches the assignment and simplifies validation.

# 3. Architecture decisions to lock early

- Robot platform now: Unitree Go2 in Gazebo.

- Fallback debug path: diff_robot only when needed for troubleshooting ideas.

- Primary transport first: MQTT.

- Secondary transport later: gRPC.

- Primary robot state source: /odom.

- Optional consistency source: /tf.

- First sensor to integrate: IMU.

- First HLA object goal: one robot object with pose, velocity, timestamp, and sequence number.

# 4. Phase 1 - Go2 simulation baseline

- Use the Unitree Go2 ROS 2 / Gazebo repository as the simulation bootstrap, not as the final project architecture.

- Create a repeatable local demo where Go2 launches in Gazebo, teleoperation works, and the core ROS interfaces are visible.

- Validate /odom, /tf, /tf_static, /imu, and at least one LiDAR topic before attempting transport or HLA integration.

- The deliverable of this phase is a stable local simulation that can be teleoperated and inspected in RViz.

# 5. Phase 2 - Canonical internal data model

- Define an internal RobotState that is independent of MQTT, gRPC, or HLA details.

- RobotState should initially include robot_id, timestamp_ns, seq, pose, quaternion, yaw, and linear/angular velocity.

- Define a separate SensorState structure so sensors can be added incrementally without destabilizing the base state pipeline.

- Start with sensor summaries rather than heavy raw data streams for LiDAR and camera.

# 6. Phase 3 - Clean ROS 2 adapter

- Write a new adapter node from scratch instead of cloning the previous team's structure.

- Adapter v1 should subscribe only to /odom, convert the message into RobotState, and log timing information.

- Keep CSV logging, timestamps, and sequence numbering because those ideas are already useful in the earlier prototype.

- Avoid embedding broker credentials, frame assumptions, or HLA mapping directly inside the adapter logic.

# 7. Phase 4 - Transport layer, MQTT first

- Add transport as a thin serialization and publish layer after the adapter works locally.

- Use clear scalable topic naming such as robot/<robot_id>/state, robot/<robot_id>/command, and robot/<robot_id>/ack.

- Use JSON first for easy inspection and debugging, then consider protobuf later if needed.

- Before touching HLA, prove that the transport path is stable with a debug subscriber and clear logs.

# 8. Phase 5 - Gateway service

- Build the gateway as a separate service whose first job is to receive, validate, normalize, and log incoming RobotState data.

- In debug mode, the gateway should run without HLA so transport and timing can be tested independently.

- Only once gateway ingest is stable should HLA publication be enabled.

- Reuse ideas such as stale packet detection, timing analysis, and optional dead reckoning, but not the inherited platform-specific implementation details.

# 9. Phase 6 - HLA / FOM mapping

- Start with one minimal object in the FOM that represents the robot.

- Initial HLA attributes should include entityId, timestamp, seq, pose, yaw, quaternion, linear velocity, angular velocity, and link or status fields if useful.

- Delay FOM extension for richer sensors until the minimal pose/velocity publication works end to end.

- Create and maintain a written mapping table: ROS topic -> internal field -> transport payload -> HLA attribute.

# 10. Phase 7 - Reverse command path

- Add return commands only after one-way telemetry to HLA is proven.

- Use a very small initial command set such as STOP, SLOW, and CLEAR or RESUME, and keep that vocabulary consistent across all components.

- Retain the ACK concept from the earlier prototype because it is useful for timing analysis and debugging command application.

# 11. Phase 8 - Sensor extension

- Integrate IMU first because it is lightweight and directly relevant to the assignment.

- Then add a LiDAR summary path, for example nearest obstacle distance, scan statistics, or a reduced-rate derived representation.

- Only add camera content if there is time and a clear use case, and prefer metadata or summarized outputs instead of full raw image transport in the first version.

# 12. Milestones

- Milestone 1: local Go2 simulation works in Gazebo with teleop and visible core topics.

- Milestone 2: new /odom-based adapter publishes RobotState locally and logs CSV.

- Milestone 3: MQTT transport path works and a debug subscriber can inspect messages.

- Milestone 4: gateway debug mode works without HLA.

- Milestone 5: HLA publication works and a viewer shows the robot moving.

- Milestone 6: reverse command path works with acknowledgements.

- Milestone 7: IMU and at least one additional sensor summary are integrated.

- Milestone 8: latency/jitter analysis and documentation are complete.

# 13. What to reuse from the previous implementation

- Useful ideas to keep: CSV timing logs, sequence numbers, stale packet detection, ACK messages, HLA viewer concepts, and optional safety logic as a later feature.

- Ideas to reuse carefully: topic naming, status fields, command bridge behavior, and already defined HLA attributes.

- Better to rebuild cleanly: adapter core, launch flow, command vocabulary, cross-platform JVM setup, configuration handling, and hard-coded environment assumptions.

# 14. Main risks

- Trying to debug Gazebo, ROS 2, transport, gateway, HLA, and sensors all at once.

- Starting from inherited code without first defining a clean architecture.

- Extending the FOM too early before the minimal HLA path works.

- Pushing heavy sensor data too soon instead of using summaries first.

- Letting platform-specific HLA runtime assumptions block progress on Ubuntu-side development.

# 15. Recommended first execution order

- Step 1: get the Go2 Gazebo simulation running locally and validate the expected ROS interfaces.

- Step 2: build a tiny new /odom-based adapter node that writes RobotState to console and CSV.

- Step 3: add MQTT publication of RobotState.

- Step 4: add a small MQTT debug subscriber and inspect message quality.

- Step 5: implement a gateway debug receiver with validation and logging.

- Step 6: connect the gateway to HLA publication.

- Step 7: add the reverse command path.

- Step 8: add IMU and then LiDAR summaries.

# 16. Proposed week 1 focus

A practical week 1 focus keeps the project moving without overcommitting to transport or HLA too early.

| Task | Expected output |
| --- | --- |
| Launch Go2 in Gazebo | Stable simulation with teleop and visible /odom, /tf, /imu |
| Inspect ROS interfaces | Confirmed topic names, frame tree, and sensor availability |
| Implement adapter skeleton | New ROS 2 node subscribed to /odom only |
| Add CSV logging | Sequence, timestamps, pose, and conversion latency written to file |
| Define internal schema | RobotState draft and field mapping note |

Bottom line: first prove Go2 Gazebo + ROS 2 works, then build a clean /odom-based adapter, then add MQTT, then a gateway debug receiver, then HLA publication, and only after that add reverse commands and richer sensors.